Stryker Helpful hints

With the new Stryker Power Pro being distributed to all AMR units, here are a few hints for ease of use and some must do’s:

HINTS

- There are only two working button on the activation pads (-) for raising the carriage and (+) for lowering/expanding the carriage.

- The head section (behind back rest) is fully retractable into the frame giving 360 degree turning radius when retracted.

- The legs lift into a true shock position, lifting from the hips

- The rear of the back rest has a monitor/possessions hook

- Can hold up to 700 lbs and has a battery life of 20 transports

- There still needs to have one person in front and rear when raising and lowering the gurney to ensure a safe trip, which means proper body mechanics need to be utilized. Use your legs.

- Make sure the magnetic device is aligned with the gurney correctly while in the ambulance, if not the gurney can be raised inside the unit…..ouch

MUST DO’S

- There must be an AMR employee on the controls at all times

- When putting in or removing from the ambulance with a patient on the gurney there must be two people at the foot (business end) of the gurney at all times.

- Patients must have all restraints in place during transport unless medically necessary to remove, after procedure completed replace restraints.

- When rolling a patient there must be a minimum of 3 hands on the gurney at ALL times, recommended to lower the patient a few inches to lower the center of gravity.

- With two people at the foot of the gurney, make sure the gurney is coming out straight. The hook will not be in plain sight to arrest the gurney prior to finding the bumper.